GLOBAL OPHTHALMIC PRIVATE LIMITED

Official Product Catalog & Technical Specifications Guide

Manufacturer Details & Source Information: • Website: https://www.globalophthalmic.com/ • Location: Plot No.3, 1st Main Road, Sri Ramsamaji Nagar Extn. Vellanoor, Ambattur, Chennai - 600062, India • GST No: 33AADCG6137A1Z6 ¦ Telephone: +91-8047646754 • Total Products Documented: 33 items across 9 core ophthalmic categories

Catalog Index & Overview

| # | Product Category | Total Items |
| --- | --- | --- |
| 1 | HYDROPHILIC ACRYLIC FOLDABLE IOLs | 8 Products |
| 2 | HYDROPHOBIC ACRYLIC FOLDABLE IOLs | 3 Products |
| 3 | PMMA IOLs | 7 Products |
| 4 | CAPSULAR TENSION RINGS | 5 Products |
| 5 | SPECIAL PRODUCTS | 3 Products |
| 6 | ANTERIOR CHAMBER INTRAOCULAR LENSES | 3 Products |
| 7 | Intraocular Lenses | 2 Products |
| 8 | Ophthalmic Knives | 1 Products |
| 9 | IOL DELIVERY SYSTEM | 1 Products |

CATEGORY: HYDROPHILIC ACRYLIC FOLDABLE IOLS

Online source: https://www.globalophthalmic.com/hydrophilic-acrylic-foldable-iols.html

1. Hydrophilic Foldable Acrylate

Indicative Price: Approx. Rs 5,000 / Piece   | High-Res Image: https://5.imimg.com/data5/PL/JU/GA/SELLER-4238712/fs-6025-d-500x500.jpg

Description: We are providing our clients with a wide assortment of
GLOWFOLD
Hydrophilic Acrylic Foldable IOL with enriched industrial experience and knowledge. Designed with utmost precision, the offered lens is designed using quality-proven violet blue light filtering material with the aid of sophisticated technology in synchronization with set industry standards. In addition to this, the provided lens undergoes various tests by our quality controllers, in order to ensure its flawless nature.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Lens Type | Hydrophilic Acrylic Foldable Intraocular Lenses |
| Optic diameter | 6.0 mm |
| Material | Hydrophilic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Overall Length | 12.50 mm |
| Model Name/Number | FS 6025,FS 6025D |
| Optic Size | 6.00 mm |
| Estimated AC Depth | 4.96 mm |
| Haptic Design | 0 Degree Pre-vaulted |
| Pco Protection | 360 Degree Step Square Edge on posterior Side |
| Sterilization | STEAM |
| Refractive Index | 1.46 @ 20 Degree C |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps,-5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Optic Design | Biconvex |
| Estimated A. constant | 118.0 |

Features & Key Advantages:

Ultra-Pure Hydrophilic Acrylic Material with adequate UV blocking.

26% Water Content.

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEE's migration.

Smooth lens surface reduces bacterial adhesion, inflammatory cell response and haptics for ease of unfolding.

Pre-vaulted design – continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design – Stable and Perfect centration and ultimate stability in the capsular bag.

Optics are YAG Laser Compatible.

Steam sterilization does away with the hazards of ETO residue.

2. ACCURAVUE acrylic foldable

Indicative Price: Approx. Rs 8,000 / Piece   | High-Res Image: https://5.imimg.com/data5/GU/QP/RJ/SELLER-4238712/accuravue-yellow-aspheric-hydrophilic-acrylic-foldable-intraocular-lenses-500x500.jpg

Description: Accura vue
is yellow series acrylic foldable IOL’s made up of FDA approved highly bio-compatible ultra pure monomer with UV-A blocking and violet blue light filtering material which contains the same UV-A blocking and violet Blue light filtering chromophore that is in human crystalline lens.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Type | Aspheric IOLs |
| Material | Hydrophilic Foldable Acrylate |
| Usage/Application | Eye diseases/ Cataract Surgery |
| Optic Design | Biconvex |
| Lens Type | Hydrophilic Acrylic Foldable Intraocular Lenses |
| Model Name/Number | YA 5725D,YA 6025,YA 6025D,YA 6030,YA 6010,YA 6005 |
| Estimated AC Depth | 4.96 mm |
| Sterilization | STEAM |
| PCO Protection | 360 Degree Step Square Edge on posterior Side |

Features & Key Advantages:

Aberration Neutral Aspheric optics-negative spherical aberration -0.27* wave front technology.

Excellent quality of vision and greatly increased contrast sensitivity. Peak visual performance in night vision and decreased glare.

Hydrophobic surface optics helps delayed PCO and well polished haptic for ease of unfolding.

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEE's migration.

Pre-vaulted haptic design-continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design-Stable and Perfect centration and ultimate stability in the capsular bag.

Optics are YAG Laser Compatible.

ST Steam sterilization does away with the hazards of ETO residue.

3. CFRANCE FOLD

Indicative Price: Approx. Rs 5,000 / Piece   | High-Res Image: https://5.imimg.com/data5/HA/AQ/OT/SELLER-4238712/fs-6025d-500x500.jpg

Description: We are providing our clients with a wide assortment of
CFRANCE FOLD
Hydrophilic Acrylic Foldable IOLs with enriched industrial experience and knowledge. Designed with precision, the offered lens is designed using quality-proven violet blue light filtering material with the aid of sophisticated technology in synchronization with set industry standards. In addition to this, the provided lens undergoes various tests by our quality controllers, in order to ensure its flawless nature.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Type | Aspheric IOLs |
| Material | Hydrophilic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Biconvex |
| Lens Type | Hydrophilic Acrylic Foldable Intraocular Lenses |
| Model Name/Number | FS 6025D |
| Optic Size | 6.00 mm |
| Estimated AC Depth | 4.96 mm |
| Sterilization | STEAM |
| Overall Length | 12.50 mm |
| Pco Protection | 360 Degree Step Square Edge on posterior Side |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps |

Features & Key Advantages:

Ultra-Pure      Hydrophilic Acrylic Material with adequate UV blocking.

26%      Water Content.

360°      Square edge design provides uninterrupted contact at the haptic-optic      junction to limit LEE's migration.

Smooth      lens surface reduces bacterial adhesion, inflammatory cell response and      haptics for ease of unfolding.

Pre-vaulted      design – continuous posterior surface contact with the capsular bag to      prevent PCO.

The      advanced IOL design – Stable and Perfect centration and ultimate stability      in the capsular bag.

Optics      are YAG Laser Compatible

Steam      sterilization does away with the hazards of ETO residue.

4. FLEXIOL AO

Indicative Price: Approx. Rs 5,000 / Piece   | High-Res Image: https://5.imimg.com/data5/KU/UC/WH/SELLER-4238712/flexiol-ao-fa6025d-500x500.jpg

Description: Flexiol AO
is an Aspheric acrylic Foldable IOL’s made of FDA approved Highly Bio-compatible ultra-pure monomer with low refractive index and blocking material.
Our Aspheric IOL’s designed with negative aberration neutral optics with 360
o
Advanced step square edge for superior optical performance and defence against PCO. Lower refractive index which minimizes problems with glare, external and internal reflections and other unwanted visual phenomena.
Our Aspheric IOL’s to be safely implanted in all patients whatever their corneal spherical aberrations.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Type | Aspheric IOLs |
| Material | Hydrophilic Foldable Acrylate |
| Usage/Application | Eye diseases/ Cataract Surgery |
| Lens Type | Hydrophilic Acrylic Foldable Intraocular Lenses |
| Model Name/Number | FFA 6025,FFA 6025D |
| Estimated AC Depth | 4.96 mm |
| Sterilization | STEAM |
| PCO Protection | 360 Degree Step Square Edge on posterior Side |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps,-5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Haptic design | 0 Degree Pre-vaulted |
| Optic Size | FFA 6025,FFA 6025D - 6.00 mm |
| Overall length | FFA 6025,FFA 6025D - 12.50 mm |
| Optic Design | Biconvex-Aberration neutral Negative Aspheric Optic |

Features & Key Advantages:

Aberration Neutral Aspheric optics-negative spherical aberration -0.27* wave front technology.

Excellent quality of vision and greatly increased contrast sensitivity. Peak visual performance in night vision and decreased glare.

Hydrophobic surface optics helps delayed PCO and well-polished haptic for ease of unfolding.

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEE's migration.

Pre-vaulted haptic design-continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design-Stable and Perfect centration and ultimate stability in the capsular bag.

Optics are YAG Laser Compatible.

ST Steam sterilization does away with the hazards of ETO residue.

5. ASPROLENZ

Indicative Price: Approx. Rs 7,000 / Piece   | High-Res Image: https://5.imimg.com/data5/OZ/DN/KG/SELLER-4238712/asprolenz-500x500.jpg

Description: Aspro lenz
is an aspheric acrylic foldable IOL’s made of FDA approved highly bio-compatible ultra pure monomer with low refractive index and blocking material.
Our Aspheric IOL’s designed with negative aberration neutral optics with 360
o
Advanced step square edge for superior optical performance and defence against PCO. Lower refractive index which minimizes problems with glare, external and internal reflections and other unwanted visual phenomena.
Our Aspheric IOL’s to be safely implanted in all patients whatever their corneal spherical aberrations.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | Hydrophilic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Brand | HydroPhillic Aspheric Foldable intraocular lenses |
| Optic Design | Biconvex-Aberration neutral Negative Aspheric Optic |
| Lens Type | Iol Intraocular Lenses |
| Model Name/Number | FA 5725D,FA 6025,FA 6025D,FA 6030,FA 6010,FA 6005 |
| Estimated AC Depth | 4.96 mm |
| Optic Size | FA 5725D - 5.75 mm,FA 6025,FA 6025D,FA 6030,FA 6010,FA 6005 - 6.00 mm |
| Sterilization | STEAM |
| Refractive Index | 1.46 @ 20 Degree C |
| Overall Length | FA 5725D,FA 6025,FA 6025D - 12.50 mm,FA 6030 - 13.00 mm,FA 6010 - 11.00 mm,FA 6005 - 10.50 mm |
| Pco Protection | 360 Degree Step Square Edge on posterior Side |
| Haptic Design | 0 Degree Pre-vaulted |
| Type | Aspheric IOLs |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps |
| Estimated A.constant | 118.0 |

Features & Key Advantages:

Aberration Neutral Aspheric optics –negative spherical aberration -0.18* wavefront technology.

Excellent quality of vision and greatly increased contrast sensitivity.

Hydrophobic surface optics helps delayed PCO and well polished haptics for ease of unfolding.

360oStep square edge design provides uninterrupted contact at haptic-optic junction to limit LEC’s migration.

Pre-vaulted haptic design – continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design – stable and perfect centration and ultimate stability in the capsular bag.

Optics is YAG laser compatible.

Steam sterilization does away with the hazards of ETO residue.

6. Flexiol

Indicative Price: Approx. Rs 5,000 / Piece   | High-Res Image: https://5.imimg.com/data5/KF/VD/ZI/SELLER-4238712/flexiol-500x500.jpg

Description: With enriched industrial experience and knowledge, we are providing our clients with a wide assortment of
Flexiol
Hydrophilic Acrylic Foldable IOLs. Designed with utmost precision, the offered lens is designed using quality-proven violet blue light filtering material with the aid of sophisticated technology in synchronization with set industry standards. In addition to this, the provided lens undergoes various tests by our quality controllers, in order to ensure its flawless nature.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | Hydrophilic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Biconvex |
| Lens Type | Hydrophilic Acrylic Foldable Intraocular Lenses |
| Optic Size | 6.00 mm |
| Overall Length | 12.50 mm |
| Pco Protection | 360 Degree Step Square Edge on posterior Side |
| Lens Model | FFS 6025,FFS 6025D |
| Haptic Design | 0 Degree Pre-vaulted |
| Available Powers | +10.0 D to +30.0 D in 0.5 D steps,-5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |
| Estimated A. constant | 118.0 |
| Estimated AC. Depth | 4.96 mm |

Features & Key Advantages:

Clear Hydrophilic

26% Water Content

360° Advanced Square edge

Ultra-Pure Hydrophilic Acrylic Material with adequate UV blocking.

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEE's migration.

Smooth lens surface reduces bacterial adhesion, inflammatory cell response and haptics for ease of unfolding.

Pre-vaulted design – continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design – Stable and Perfect centration and ultimate stability in the capsular bag.

Optics are YAG Laser Compatible

Steam sterilization does away with the hazards of ETO residue.

7. HEMAFOLD Hydrophilic Acrylic

Indicative Price: Approx. Rs 5,000 / Piece   | High-Res Image: https://5.imimg.com/data5/WA/PQ/GX/SELLER-4238712/fs-6025-d-500x500.jpg

Description: Product Brochure
With enriched industrial experience and knowledge, we are providing our clients with a wide assortment of
Hema Fold
Hydrophilic Acrylic Foldable IOL. Designed with utmost precision, the offered lens is designed using quality-proven violet blue light filtering material with the aid of sophisticated technology in synchronization with set industry standards. In addition to this, the provided lens undergoes various tests by our quality controllers, in order to ensure its flawless nature.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | Hydrophilic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Model Name/Number | FS 5725D,FS 6025,FS 6025D,FS 6030,FS 6005 |
| Estimated AC Depth | 4.96 mm |
| Sterilization | STEAM |
| Refractive Index | 1.46 @ 20 Degree C |
| PCO Protection | 360 Degree Step Square Edge on posterior Side |
| Optic Design | Biconvex |

Features & Key Advantages:

Clear Hydrophilic

26% Water Content

360° Advanced Square edge

Ultra Pure Hydrophilic acrylic Material with adequate UV blocking

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEE's migration

Smooth lens surface reduces bacterial adhesion, inflammatory cell response and haptics for ease of unfolding

Pre-vaulted design – continuous posterior surface contact with the capsular bag to prevent PCO

The advanced IOL design – Stable and Perfect centration and ultimate stability in the capsular bag

Optics are YAG Laser Compatible

Steam sterilization does away with the hazards of ETO residue.

8. Hydrophilic OPTIOL FOLD

Indicative Price: Approx. Rs 5,000 / Piece   | High-Res Image: https://5.imimg.com/data5/VE/RX/OZ/SELLER-4238712/ofs-6025-d-500x500.jpg

Description: We are providing our clients with a wide assortment of
OPTIOL FOLD
Hydrophilic Acrylic Foldable IOLs with enriched industrial experience and knowledge. Designed with precision, the offered lens is designed using quality-proven violet blue light filtering material with the aid of sophisticated technology in synchronization with set industry standards. In addition to this, the provided lens undergoes various tests by our quality controllers, in order to ensure its flawless nature.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Lens Type | Hydrophilic Acrylic Foldable Intraocular Lenses |
| Material | Hydrophilic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Overall Length | 12.50 mm |
| Optic Size | 6.00 mm |
| Model Name/Number | OFS 6025, OFS 6025D |
| Estimated AC Depth | 4.96 mm |
| Haptic Design | 0 Degree Pre-vaulted |
| Pco Protection | 360 Degree Step Square Edge on posterior Side |
| Sterilization | STEAM |
| Refractive Index | 1.46 @ 20 Degree C |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps, -5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Optic Design | Biconvex |
| Estimated A. constant | 118.0 |

Features & Key Advantages:

Ultra-Pure      Hydrophilic Acrylic Material with adequate UV blocking.

26%      Water Content.

360°      Square edge design provides uninterrupted contact at the haptic-optic      junction to limit LEC's migration.

Smooth      lens surface reduces bacterial adhesion, inflammatory cell response and      haptics for ease of unfolding.

Pre-vaulted      design – continuous posterior surface contact with the capsular bag to      prevent PCO.

The      advanced IOL design – Stable and Perfect centration and ultimate stability      in the capsular bag.

Optics      are YAG Laser Compatible

Steam      sterilization does away with the hazards of ETO residue.

CATEGORY: HYDROPHOBIC ACRYLIC FOLDABLE IOLS

Online source: https://www.globalophthalmic.com/hydrophobic-acrylic-foldable-iols.html

9. FLEXIOL PHOBIC Aspheric

Indicative Price: Approx. Rs 9,000 / Piece   | High-Res Image: https://5.imimg.com/data5/VS/BF/BH/SELLER-4238712/flexiol-phobic-500x500.jpg

Description: FLEXIOL PHOBIC
Aspheric Hydrophobic foldable IOL made from pure acrylic polymer and a proprietary cryo-lathing process with no glistening. Flexiol Phobic Aspheric series intraocular lenses with aberration free Ultra view advanced technology. Our Aspheric IOL’s with negative spherical aberration to compensate for positive aberration of an average cornea.
Our Negative spherical aberration aspheric design results to reduce or eliminate the spherical aberration of the eye and peak visual performance like young adults after cataract surgery with our lens. Our foldable acrylic IOLs are lower refractive index- which minimizes problems with glare, external and internal reflections and other unwanted visual phenomena. Our Aspheric IOL’s to be safely implanted whatever their corneal spherical aberrations.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Type | Aspheric IOLs |
| Material | Hydrophobic Foldable Acrylate |
| Usage/Application | Eye Diseases / Cataract Surgery |
| Lens Type | HYDROPHOBIC ACRYLIC FOLDABLE INTRAOCULAR LENSES |
| Model Name/Number | FHA 6025,FHA 6030 |
| Estimated AC Depth | 5.1 mm |
| Optic Size | 6.00 mm |
| Sterilization | Ethylene Oxide |
| Lens Style | Single Piece Acrylic Foldable Intraocular lens |
| Refractive Index | 1.49 |
| Overall Length | FHA 6025 - 12.50 mm,FHA 6030 - 13.00 mm |
| Pco Protection | 360 Degree Step Square Edge on posterior Side |
| Haptic Design | 5 Degree |
| Available Powers | + 4.0 D to +35.0 D in 0.5 D steps |
| Material Description | Co-polymer of Phenylethyl acrylate (PEA) and Phenylethyl methacrylate (PEMA) which is cross-linked w |

Features & Key Advantages:

Aberration Neutral Aspheric optics-negative spherical aberration wave front technology.

Excellent quality of vision and greatly increased contrast sensitivity.

Well polished smooth Hydrophobic optics helps delayed PCO and haptic for ease of unfolding.

360oSquare edge design provides uninterrupted contact at the haptic-optic junction to limit LEC’s migration.

5oAngulations haptic design – continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design-Stable and Perfect centration and ultimate stability in the capsular bag.

Optics is YAG Laser Compatible.

10. HYPHOFLEX

Indicative Price: Approx. Rs 10,500 / Piece   | High-Res Image: https://5.imimg.com/data5/PX/YA/DN/SELLER-4238712/hyphoflex-hydrophobic-intraocular-lens-500x500.jpg

Description: Product Brochure
HYPHOFLEX
Aspheric Hydrophobic foldable IOL made from pure acrylic polymer and a proprietary cryo-lathing process with no glistening. HYPHOflex Aspheric series intraocular lenses with aberration free Ultra view advanced technology. Our Aspheric IOL’s with negative spherical aberration to compensate for positive aberration of an average cornea. Our Negative spherical aberration aspheric design results to reduce or eliminate the spherical aberration of the eye and peak visual performance like young adults after cataract surgery with our lens. Our foldable acrylic IOLs are lower refractive index- which minimizes problems with glare, external and internal reflections and other unwanted visual phenomena. Our Aspheric IOL’s to be safely implanted whatever their corneal spherical aberrations.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Types | Aspheric IOLs, Monovision |
| Type | Aspheric IOLs |
| Usage/Application | Eye Diseases / Cataract surgery |
| Lens Type | Hydrophobic Intraocular lenses |
| Material | Hydrophobic Foldable Acrylate |
| Power | +10D To 25D are available |
| Optic Size | 6.00 MM |
| Refractive Index | 1.56 |
| Overall Length | 12.50 mm & 13.00 mm |
| PCO Protection | Square edge are available |
| Lens Model | All Types are available |
| Haptic Design | Modified C Haptic |
| Available Powers | + 4.0 D to +35.0 D in 0.5 D steps |
| Sterilization | Yes |

Features & Key Advantages:

Aberration Neutral Aspheric optics-negative spherical aberration wave front technology.

Excellent quality of vision and greatly increased contrast sensitivity.

Well polished smooth Hydrophobic optics helps delayed PCO and haptic for ease of unfolding.

360oSquare edge design provides uninterrupted contact at the haptic-optic junction to limit LEC’s migration.

5oAngulations haptic design – continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design-Stable and Perfect centration and ultimate stability in the capsular bag.

Optics is YAG Laser Compatible.

11. Hyphovue

Indicative Price: Approx. Rs 15,000 / Piece   | High-Res Image: https://5.imimg.com/data5/TG/VF/BZ/SELLER-4238712/hyphovue-500x500.jpg

Description: HYPHOvue
Yellow series acrylic foldable IOLs made from pure hydrophobic acrylic polymer with UV-A blocking and violet Blue light absorption material. This is the same UV-A blocking and Violet Blue light filtering chromosphore that is in the human crystalline lens. Blue light absorption IOLs to reduce the risk of age related macular degeneration. Blue blocking IOLs reduce low light contrast sensitivity as well as color perception. These capabilities are critically needed for optimum vision of implant patients, particularly at night. Natural Yellow achieves complete natural protection without loss of contrast sensitivity or color perception.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Type | Aspheric IOLs |
| Material | Hydrophobic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Biconvex-Aberration neutral Negative Aspheric Optic |
| Lens Type | Yellow Hydrophobic Acrylic Foldable Intraocular Lenses |
| Model Name/Number | HY 6025,HY 6030 |
| Estimated AC Depth | 5.1 mm |
| Optic Size | 6.00 mm |
| Refractive Index | 1.49 |
| Overall Length | HY 6025 - 12.50 mm,HY 6030 - 13.00 mm |
| PCO Protection | 360 Degree Step Square Edge on posterior Side |
| Haptic Design | 5 Degree |
| Light Transmission | UV-A blocking and Violet Blue light filtering |
| Available Powers | +4.0 D to +35.0 D in 0.5 D steps |
| Sterilization | Yes |

Features & Key Advantages:

Aberration Neutral Aspheric optics-negative spherical aberration wave front technology.

Excellent quality of vision and greatly increased contrast sensitivity.

Well polished smooth Hydrophobic optics helps delayed PCO and haptic for ease of unfolding.

360oSquare edge design provides uninterrupted contact at the haptic-optic junction to limit LEC’s migration.

5oAngulations haptic design – continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design-Stable and Perfect centration and ultimate stability in the capsular bag.

Optics is YAG Laser Compatible.

CATEGORY: PMMA IOLS

Online source: https://www.globalophthalmic.com/pmma-iols.html

12. PROXYLENZ Yellow Aspheric

Indicative Price: Approx. Rs 4,500 / Piece   | High-Res Image: https://5.imimg.com/data5/LU/BF/PD/SELLER-4238712/proxylenz-blf-500x500.jpg

Description: Our
PROXY LENZ Yellow Aspheric IOLs
are designed
with square edge. The 360° square edge acts as a complete barrier to epithelial cells and reduces incidence of PCO. The anterior surface of the Lens is round to avoid glare effect. We provide this lens in different sizes and designs as per the requirement of our clients.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Type | Aspheric IOLs |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Biconvex-Aberration neutral Negative Aspheric Optic |
| Lens Type | Pmma Intraocular Lenses |
| Optic Size | PY 5025 - 5.00 mm; PY 5525 - 5.50 mm; PY 6025 - 6.00 mm |
| Model Name/Number | PY 5025,PY 5525,PY 6025 |
| Overall Length | PY 5025,PY 5525,PY 6025 12.50 mm |
| Pco Protection | 360 Degree Square Edge on posterior Side |
| Haptic Design | Modified "C" step vault |
| Drilling Holes | PY 6025 - YES; PY 5025,PY 5525 - -Nil- |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps |
| Sterilization | Yes |

Features & Key Advantages:

Smooth lens surface

Reduced bacterial adhesion, inflammatory cell response

Step-vaulted haptic design

Highly biocompatible Clear PMMA-CQ with adequate UV blocking.

Aberration Natural Aspheric Biconvex optics. Excellent quality of vision, greatly increased contrast sensitivity and reduced glare.

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEE's migration.

Smooth lens surface reduces bacterial adhesion, inflammatory cell response.

Step-vaulted haptic design continuous posterior surface contact with the capsular bag to prevent PCO.

Optics are YAG Laser Compatible.

Sterilized by Ethylene Oxide.

13. GLOWVISION

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/PO/OD/EC/SELLER-4238712/glowvision-500x500.jpg

Description: Product Brochure
These are made up of high quality PMMA lens. It is placed behind the iris in the capsule. Implantation inside the capsular bag (first choice) or fix into the sulcus if the capsular bag is damaged. It is the widely used IOL for cataract surgery.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Optic Design | Spherical Bi-convex |
| Lens Type | Pmma Intraocular Lenses |
| Type | POSTERIOR CHAMBER INTRAOCULAR LENSES |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Material | PMMA |
| Model Name/Number | PS 5025; PS 5220; PS 6025; PS 6030; PS 6530; PS 6535 |
| Haptic Design | "C" design with Step Vault |
| Overall Diameter | PS 5025,PS 6025 - 12.50 mm; PS 6030,PS 6530 - 13.00 mm; PS 5220 - 12.00 mm; PS 6535 - 13.50 mm |
| Drilling Holes | PS 6025,PS 6030,PS 6530 - Yes PS 5025,PS 5220 - -Nil- |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps; -5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |

Features & Key Advantages:

Larger optic diameter.

Improved the PCO protection efficiency of the square edge.

Increased stability.

Clear vision.

14. GLOWEDGE

Indicative Price: Approx. Rs 4,000 / Piece   | High-Res Image: https://5.imimg.com/data5/TZ/AE/FA/SELLER-4238712/glowedge-500x500.jpg

Description: GLOWEDGE
Aspheric IOLs are made of PMMA material. It has 360° sharp edge on the posterior side of the lens and aspheric optic on anterior surface. Glowedge lens are designed with 360° sharp edge on the posterior side of the lens and aspheric optic with zero aberration on the anterior side of the IOL. This sharp edge of the IOL prevents the epithelial cells migrating to the posterior side, thereby reducing the PCO (Posterior capsule opacification) rate.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Optic Design | Biconvex |
| Lens Type | Pmma Intraocular Lenses |
| Type | Aspheric IOLs |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Refractive Index | 1.49 C |
| Model Name/Number | PE 5525,PE 6025 |
| Overall Length | 12.50 mm |
| Pco Protection | 360 Degree Square Edge on posterior Side |
| Haptic Design | Modified "C" step vault |
| Optic Diameter | PE 5525 - 5.50 mm; PE 6025 - 6.00 mm |
| Drilling Holes | PE 6025 - YES; PE 5525 - -Nil- |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps |
| Sterilization | Yes |

Features & Key Advantages:

Highly biocompatible Clear PMMA-CQ with adequate UV blocking.

Aberration Natural Aspheric Biconvex optics. Excellent quality of vision, greatly increased contrast sensitivity and reduced glare.

360°Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEE's migration.

Smooth lens surface reduces bacterial adhesion, inflammatory cell response.

Step-vaulted haptic design continuous posterior surface contact with the capsular bag to prevent PCO.

Optics are YAG Laser Compatible.

Sterilized by Ethylene Oxide.

15. PROXYLENZ

Indicative Price: Approx. Rs 4,000 / Piece   | High-Res Image: https://5.imimg.com/data5/NL/RL/ZT/SELLER-4238712/proxy-lenz-aspheric-pmma-iol-pa5225-500x500.jpg

Description: Our PROXY lenz Aspheric IOLs are designed with square edge. The 360° square edge acts as a complete barrier to epithelial cells and reduces incidence of PCO. The anterior surface of the Lens is round to avoid glare effect. We provide this lens in different sizes and designs as per the requirement of our clients.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Type | Aspheric IOLs |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Biconvex-Aberration neutral Negative Aspheric Optic |
| Lens Type | Pmma Intraocular Lenses |
| Optic Size | PA 5025 - 5.00 mm; PA 5220 - 5.25 mm; PA 5525 - 5.50 mm; PA 6025 - 6.00 mm |
| Refractive Index | 1.49 C |
| Overall Length | PA 5025,PA 5525,PA 6025 - 12.50 mm; PA 5220 - 12.00mm |
| Pco Protection | 360 Degree Square Edge on posterior Side |
| Drilling Holes | PA 6025 - Yes; PA 5025,PA 5220,PA 5525 - -Nil- |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps |
| Sterilization | Yes |

Features & Key Advantages:

Highly biocompatible Clear PMMA-CQ with adequate UV blocking

Aberration Natural Aspheric Biconvex optics

Excellent quality of vision, greatly increased contrast sensitivity and reduced glare

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEC\'s migration

Smooth lens surface reduces bacterial adhesion, inflammatory cell response

Step-vaulted haptic design continuous posterior surface contact with the capsular bag to prevent PCO

Optics are YAG Laser Compatible

Sterilized by Ethylene Oxide

16. Optiol Gold

Indicative Price: Approx. Rs 4,500 / Piece   | High-Res Image: https://5.imimg.com/data5/AG/RY/PY/SELLER-4238712/optiol-gold-500x500.jpg

Description: Our
OPTIOL GOLD Yellow Aspheric IOLs
are designed with square edge. The 360° square edge acts as a complete barrier to epithelial cells and reduces incidence of PCO. The anterior surface of the Lens is rounded to avoid glare effect. We provide this lens in different sizes and designs as per the requirement of our clients.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Type | Aspheric IOLs |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Biconvex-Aberration neutral Negative Aspheric Optic |
| Lens Type | Pmma Intraocular Lenses |
| Optic Size | OPY 5525 - 5.50 mm; OPY 6025 - 6.00 mm |
| Overall Length | OPY 5525,OPY 6025 - 12.50 mm; |
| Pco Protection | 360 Degree Square Edge on posterior Side |
| Lens Model | OPY 5525,OPY 6025 |
| Haptic Design | Modified "C" step vault |
| Drilling Holes | OPY 6025 - Yes ; OPY 5525 - -Nil- |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps,-5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |

Features & Key Advantages:

Smooth lens surface.

Reduced bacterial adhesion, inflammatory cell response.

Step-vaulted haptic design.

Aberration Natural Aspheric Biconvex optics. Excellent quality of vision, greatly increased contrast sensitivity and reduced glare.

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEE's migration.

Smooth lens surface reduces bacterial adhesion, inflammatory cell response.

Step-vaulted haptic design continuous posterior surface contact with the capsular bag to prevent PCO.

Optics are YAG Laser Compatible.

Sterilized by Ethylene Oxide.

17. Catracure

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/RL/NT/GQ/SELLER-4238712/catracure-500x500.jpg

Description: Catracure IOLs
are made up of high quality PMMA lens. It is placed behind the iris in the capsule. Implantation inside the capsular bag (first choice) or fix into the sulcus if the capsular bag is damaged. It is the widely used IOL for cataract surgery.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 1300 Piece |
| Type | Posterior Chamber IOLs |
| Material | PMMA |
| Usage/Application | Eye Diseases |
| Optic Design | Spherical Bi-convex |
| Lens Type | Pmma Intraocular Lenses |
| Overall Length | CPS 5525,CPS 6025 - 12.50 mm |
| Model Name/Number | CPS 5525,CPS 6025 |
| Optic Diameter | CPS 5525 - 5.50 mm; CPS 6025 - 6.00 mm |
| Drilling Holes | CPS 6025 - Yes; CPS 5525 - -Nil- |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps; -5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |

Features & Key Advantages:

Larger optic diameter.

Improved the PCO protection efficiency of the square edge.

Increased stability.

Clear vision.

18. Iovue

Indicative Price: Approx. Rs 120 / Piece   | High-Res Image: https://5.imimg.com/data5/CR/RF/TN/SELLER-4238712/ips-6025-500x500.jpg

Description: These are made up of high quality PMMA lens. It is placed behind the iris in the capsule. Implantation inside the capsular bag (first choice) or fix into the sulcus if the capsular bag is damaged. It is the widely used IOL for cataract surgery.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Optic Design | Spherical Bi-convex |
| Lens Type | Pmma Intraocular Lenses |
| Optic Size | IPS 5525 - 5.50 mm; IPS 6025 - 6.00 mm |
| Usage/Application | / Cataract Surgery |
| Material | PMMA |
| Model Name/Number | IPS 5525,IPS 6025 |
| Overall Length | IPS 5525,IPS 6025 - 12.50 mm |
| Haptic Design | "C" design with Step Vault |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps,-5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |

Features & Key Advantages:

Larger optic diameter

Step-vaulted haptic design continuous posterior surface contact with the capsular bag to prevent PCO

Increased stability

Clear vision

Optics are YAG Laser Compatible

Sterilized by Ethylene Oxide

CATEGORY: CAPSULAR TENSION RINGS

Online source: https://www.globalophthalmic.com/capsular-tension-rings.html

19. Capsular Tension Rings(CTR) (Model: PR 1109,PR 1210)

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/UY/MS/IN/SELLER-4238712/capsring-500x500.jpg

Description: Capsring
prevents vitreous herniation and maintain the circular contour of the capsular bag. Capsring also tend to reduce intraocular lens de-centralization, which improves vision and gives additional support to the IOL. They may also inhibit epithelial cell proliferation, thus reducing the chances of capsule opacification.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | PMMA |
| Brand | CAPSRING |
| Sterile | Ethylene Oxide Sterilization |
| Model Name/Number | PR 1109,PR 1210 |
| Color | Blue |
| Packaging Type | Box Packing |
| Size/Dimension | PR 1109 - 11.00 mm; PR 1210 - 12.00 mm |
| Usage/Application | Hospital |
| Insertion Size | PR 1109 - 9.00 mm; PR 1210 - 10.00 mm |

Features & Key Advantages:

PMMA-CQ with UV filter Clear and Blue colours available.

Advanced Capsular tension rings are single or double fixation hooks with step vaulted, specially designed for scleral fixation with suture.

Circular expansion and stabilization of the periphery of the capsular bag.

Reduced risk of capsular fibrosis and shrinking.

Safe IOL centration in eye with zonular dehiscence.

20. Caps Ring (Model: PR 1210L,PR 1311L)

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/TD/QX/PE/SELLER-4238712/pr-1210l-500x500.jpg

Description: Capsring
prevents vitreous herniation and maintain the circular contour of the capsular bag. Capsring also tend to reduce intraocular lens de-centralization, which improves vision and gives additional support to the IOL. They may also inhibit epithelial cell proliferation, thus reducing the chances of capsule opacification.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | PMMA |
| Brand | CAPSRING |
| Sterile | Ethylene Oxide Sterilization |
| Model Name/Number | PR 1210L,PR 1311L |
| Color | Blue |
| Size/Dimension | PR 1210L - 12.00 mm; PR 1311L - 13.00 mm |
| Usage/Application | Hospital |
| Country of Origin | Made in India |
| Insertion Size | PR 1210L - 10.00 mm; PR 1311L - 11.00 mm |

Features & Key Advantages:

PMMA-CQ with UV filter Clear and Blue colours available.

Advanced Capsular tension rings are single or double fixation hooks with step vaulted, specially designed for scleral fixation with suture.

Circular expansion and stabilization of the periphery of the capsular bag.

Reduced risk of capsular fibrosis and shrinking.

Safe IOL centration in eye with zonular dehiscence.

21. Capsring (Model: PR 1210R,PR 1311R)

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/TO/DD/JG/SELLER-4238712/pr-1210r-500x500.jpg

Description: Capsring
prevents vitreous herniation and maintain the circular contour of the capsular bag. Capsring also tend to reduce intraocular lens de-centralization, which improves vision and gives additional support to the IOL. They may also inhibit epithelial cell proliferation, thus reducing the chances of capsule opacification.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | PMMA |
| Brand | CAPSRING |
| Sterile | Ethylene Oxide Sterilization |
| Model Name/Number | PR 1210R,PR 1311R |
| Color | Blue |
| Packaging Type | Box Packing |
| Size/Dimension | PR 1210R - 12.00 mm; PR 1311R - 13.00 mm |
| Usage/Application | Hospital |
| Insertion Size | PR 1210R - 10.00 mm; PR 1311R - 11.00 mm |

Features & Key Advantages:

PMMA-CQ with UV filter Clear and Blue colours available.

Advanced Capsular tension rings are single or double fixation hooks with step vaulted, specially designed for scleral fixation with suture.

Circular expansion and stabilization of the periphery of the capsular bag.

Reduced risk of capsular fibrosis and shrinking.

Safe IOL centration in eye with zonular dehiscence.

22. Caps Ring (Model: PR 1210LR,PR 1311LR)

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/TH/GA/ZM/SELLER-4238712/pr-1210lr-500x500.jpg

Description: Capsring
prevents vitreous herniation and maintain the circular contour of the capsular bag. Capsring also tend to reduce intraocular lens de-centralization, which improves vision and gives additional support to the IOL. They may also inhibit epithelial cell proliferation, thus reducing the chances of capsule opacification.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | PMMA |
| Brand | CAPSRING |
| Sterile | Ethylene Oxide Sterilization |
| Model Name/Number | PR 1210LR,PR 1311LR |
| Color | Blue |
| Packaging Type | Box Packing |
| Size/Dimension | PR 1210LR - 12.00 mm; PR 1311LR - 13.00 mm |
| Usage/Application | Hospital |
| Insertion Size | PR 1210LR - 10.00 mm; PR 1311LR - 11.00 mm |

Features & Key Advantages:

PMMA-CQ with UV filter Clear and Blue colours available.

Advanced Capsular tension rings are single or double fixation hooks with step vaulted, specially designed for scleral fixation with suture.

Circular expansion and stabilization of the periphery of the capsular bag.

Reduced risk of capsular fibrosis and shrinking.

Safe IOL centration in eye with zonular dehiscence.

23. Caps Ring (Model: PR 1311,PR 1412)

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/PS/TQ/AM/SELLER-4238712/pr-1311-500x500.jpg

Description: Capsring
prevents vitreous herniation and maintain the circular contour of the capsular bag. Capsring also tend to reduce intraocular lens de-centralization, which improves vision and gives additional support to the IOL. They may also inhibit epithelial cell proliferation, thus reducing the chances of capsule opacification.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | PMMA |
| Brand | CAPSRING |
| Sterile | Ethylene Oxide Sterilization |
| Model Name/Number | PR 1311,PR 1412 |
| Color | Blue |
| Size/Dimension | PR 1311 - 13.00 mm; PR 1412 - 14.00 mm |
| Usage/Application | Hospital |
| Country of Origin | Made in India |
| Insertion Size | PR 1311 - 11.00 mm; PR 1412 - 12.00 mm |

Features & Key Advantages:

PMMA-CQ with UV filter Clear and Blue colours available.

Advanced Capsular tension rings are single or double fixation hooks with step vaulted, specially designed for scleral fixation with suture.

Circular expansion and stabilization of the periphery of the capsular bag.

Reduced risk of capsular fibrosis and shrinking.

Safe IOL centration in eye with zonular dehiscence.

CATEGORY: SPECIAL PRODUCTS

Online source: https://www.globalophthalmic.com/special-products.html

24. Hydrophobic Acrylic Multifocal/ Toric/ Multifocal Toric Foldable Intraocular Lenses

Indicative Price: Approx. Rs 25,000 / Piece   | High-Res Image: https://5.imimg.com/data5/YF/JE/CM/SELLER-4238712/hm-6030-500x500.jpg

Description: GLOBAL OPHTHALMIC Hydrophobic Foldable IOL made from ultra-high-purity hydrophobic acrylic-copolymer of Phenylethyl acrylate (PEA) and Phenylethyl methacrylate (PEMA) which is cross-linked with Butanediol diacrylate (BDDA) and does include a benzotriazole UV absorber.  Our HYDROPHOBIC IOL Negatively aspheric lens which corrects 0.25µm of spherical aberration (SA), designed to produce zero spherical aberration in an eye model representing an average normal population.
The material has been developed to create a balance between high refractive index, compression and fast unfolding time.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Type | Multifocal IOLs/ Toric IOLs/ Multifocal Toric IOLs |
| Material | Hydrophobic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Lens Type | Iol Intraocular Lenses |
| Refractive Index | 1.56 |
| Lens Model | HM 6030,HT 6030,HTM 6030,HM 6010,HTM 6010 |
| Haptic Design | Modified "C" Haptic |
| Overall Diameter | HM 6030,HT 6030,HTM 6030 - 13.00 mm; HM 6010,HTM 6010 - 11.00 mm |
| Optic Diameter | HM 6030,HT 6030,HTM 6030,HM 6010,HTM 6010 - 6.00 mm |
| Sterilization | Yes |
| Hoffer Q (pACD) | 5.11 |

Features & Key Advantages:

Aberration Neutral Aspheric optics - negative spherical aberration wave front technology.

Excellent quality of vision and greatly increased contrast sensitivity.

Well-polished smooth Hydrophobic optics helps delayed PCO and haptics for ease of unfolding.

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEC's migration.

5° Angulation haptic design - continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design - Stable and Perfect centration and ultimate stability in the capsular bag.

Optics are YAG Laser Compatible.

25. Yellow Hydrophobic Acrylic Multifocal/ Toric/ Multifocal Toric Intraocular Lenses

Indicative Price: Approx. Rs 30,000 / Piece   | High-Res Image: https://5.imimg.com/data5/BR/RH/CX/SELLER-4238712/yellow-hydrophobic-acrylic-multifocal-toric-multifocal-toric-intraocular-lenses-500x500.jpg

Description: GLOBAL OPHTHALMIC Hydrophobic Foldable IOL made from ultra-high-purity hydrophobic acrylic-copolymer of Phenylethyl acrylate (PEA) and Phenylethyl methacrylate (PEMA) which is cross-linked with Butanediol diacrylate (BDDA) and does include a benzotriazole UV absorber.  Our HYDROPHOBIC IOL Negatively aspheric lens which corrects 0.25µm of spherical aberration (SA), designed to produce zero spherical aberration in an eye model representing an average normal population.
The material has been developed to create a balance between high refractive index, compression and fast unfolding time.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Type | Multifocal IOLs/ Toric IOLs/ Multifocal Toric IOLs |
| Material | Hydrophobic Foldable Acrylate |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Lens Type | Iol Intraocular Lenses |
| Refractive Index | 1.56 |
| Lens Model | HYM 6030,HYT 6030,HYTM 6030,HYM 6010,HYTM 6010 |
| Haptic Design | Modified "C" Haptic |
| Overall Diameter | HYM 6030,HYT 6030,HYTM 6030 - 13.00 mm; HYM 6010,HYTM 6010 - 11.00 mm |
| Optic Diameter | HYM 6030,HYT 6030,HYTM 6030,HYM 6010,HYTM 6010 - 6.00 mm |
| Available Powers | +10.00 D to +30.00 D in 0.5 steps |
| Sterilization | Yes |
| Hoffer Q (pACD) | 5.11 |

Features & Key Advantages:

Aberration Neutral Aspheric optics - negative spherical aberration wave front technology.

Excellent quality of vision and greatly increased contrast sensitivity.

Well-polished smooth Hydrophobic optics helps delayed PCO and haptics for ease of unfolding.

360° Square edge design provides uninterrupted contact at the haptic-optic junction to limit LEC's migration.

5° Angulation haptic design - continuous posterior surface contact with the capsular bag to prevent PCO.

The advanced IOL design - Stable and Perfect centration and ultimate stability in the capsular bag.

Optics are YAG Laser Compatible.

26. Irisglow

Indicative Price: Approx. Rs 2,000 / Piece   | High-Res Image: https://5.imimg.com/data5/CP/WM/XT/SELLER-4238712/irisglow-500x500.jpg

Description: IRISGLOW IOLs are meant for the correction of refractive errors, providing patients with high quality vision. These are inserted either on top of the iris or in between the iris and the natural lens. They are used to treat refractive error without removing cornea tissue or the lens.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Spherical Bi-convex |
| Lens Type | Pmma Intraocular Lenses |
| Optic Size | PS 4272 - 4.25 mm,PS 5085 - 5.00 mm,PS 5580 - 5.50 mm |
| Haptic Design | "S" design with Step Vault |
| Overall Diameter | PS 4272 - 7.25 mm,PS 5085 - 8.50 mm,PS 5580 - 8.00 mm |
| Drilling Holes | -Nil- |
| Sterilization | Yes |
| A Constant | 115.3 |

Features & Key Advantages:

Improved the PCO protection efficiency of the square edge

Increased stability

Clear vision

Smooth lens surface

Available in different powers

Step-vaulted haptic design

YAG Laser Compatible

CATEGORY: ANTERIOR CHAMBER INTRAOCULAR LENSES

Online source: https://www.globalophthalmic.com/anterior-chamber-intraocular-lenses.html

27. OPTIOL Anterior chamber IOL

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/GJ/FO/UC/SELLER-4238712/oac-6025-500x500.jpg

Description: OPTIOL Anterior chamber IOL
is fixed with the support of iris. It is used as backup lens primarily when there is damage to the Posterior Chamber. Anterior Chamber IOL is also used in refractive surgery – implanting a lens in phakic eye to correct the refractive power.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Type | Anterior Chamber Intraocular Lenses |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Spherical Bi-convex |
| Lens Type | Pmma Intraocular Lenses |
| Overall Length | 12.50 mm |
| Lens Model | OAC 6025 |
| Haptic Design | "S" design with Step Vault |
| Optic Diameter | 6.00mm |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps; -5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |
| Suggested A. constant | 115.3 |

Features & Key Advantages:

Improved the PCO protection efficiency of the square edge.

Increased stability.

Clear vision.

Smooth lens surface.

Available in different powers.

Step-vaulted haptic design.

YAG Laser Compatible.

28. Catracure

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/DL/PM/CK/SELLER-4238712/cac-6025-500x500.jpg

Description: CATRACURE Anterior chamber IOL
is fixed with the support of iris. It is used as backup lens primarily when there is damage to the Posterior Chamber. Anterior Chamber IOL is also used in refractive surgery – implanting a lens in phakic eye to correct the refractive power.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Type | Anterior Chamber IOL |
| Lens Type | Pmma Intraocular Lenses |
| Design | Rigid |
| Overall Length | 12.50 mm |
| Optic Diameter | 6.00 mm |
| Haptic Design | "S" design with Step Vault |
| Lens Model | CAC 6025 |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps; -5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |
| Optic Design | Spherical Bi-convex |
| Suggested A. constant | 115.3 |

Features & Key Advantages:

Improved the PCO protection efficiency of the square edge.

Increased stability.

Clear vision.

Smooth lens surface.

Available in different powers.

Step-vaulted haptic design.

YAG Laser Compatible.

29. Glowvision

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/PX/WJ/LS/SELLER-4238712/ac-6025-500x500.jpg

Description: GLOWvision Anterior chamber IOL
is fixed with the support of iris. It is used as backup lens primarily when there is damage to the Posterior Chamber. Anterior Chamber IOL is also used in refractive surgery – implanting a lens in phakic eye to correct the refractive power.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Type | Anterior Chamber IOL |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Spherical Bi-convex |
| Lens Type | Pmma Intraocular Lenses |
| Overall Length | 12.50 mm |
| Lens Model | AC 6025 |
| Haptic Design | "S" design with Step Vault |
| Optic Diameter | 6.00mm |
| Drilling Holes | Nil |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps; -5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |
| Suggested A. constant | 115.3 |

Features & Key Advantages:

Improved the PCO protection efficiency of the square edge.

Increased stability.

Clear vision.

Smooth lens surface.

Available in different powers.

Step-vaulted haptic design.

YAG Laser Compatible.

CATEGORY: INTRAOCULAR LENSES

Online source: https://www.globalophthalmic.com/intraocular-lenses.html

30. Optiol PMMA  Lenses

Indicative Price: Approx. Rs 1,000 / Piece   | High-Res Image: https://5.imimg.com/data5/IF/VP/LY/SELLER-4238712/ops-6025-500x500.jpg

Description: These are made up of high quality PMMA lens. It is placed behind the iris in the capsule. Implantation inside the capsular bag (first choice) or fix into the sulcus if the capsular bag is damaged. It is the widely used IOL for cataract surgery.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Lens Type | Pmma Intraocular Lenses |
| Optic Size | OPS 5525 - 5.50 mm; OPS 6025 - 6.00 mm |
| Model Name/Number | OPS 5525,OPS 6025 |
| Overall Length | 12.50 mm |
| Haptic Design | "C" design with Step Vault |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps,-5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |

Features & Key Advantages:

Larger optic diameter.

Step-vaulted haptic design continuous posterior surface contact with the capsular bag to prevent PCO.

Increased stability.

Clear vision.

Optics are YAG Laser Compatible.

Sterilized by Ethylene Oxide.

31. Optitechnics PMMA Lens

Indicative Price: Approx. Rs 350 / Piece   | High-Res Image: https://5.imimg.com/data5/ZZ/WW/VT/SELLER-4238712/s6003h2c-500x500.jpg

Description: These are made up of high quality PMMA lens. It is placed behind the iris in the capsule. Implantation inside the capsular bag (first choice) or fix into the sulcus if the capsular bag is damaged. It is the widely used IOL for cataract surgery.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Minimum Order Quantity | 100 Piece |
| Color | White |
| Lens Out Diameter | All types are available |
| Material | PMMA |
| Usage/Application | Eye Diseases/ Cataract Surgery |
| Optic Design | Spherical Bi-convex |
| Lens Type | Pmma Intraocular Lenses |
| Brand | OPTITECHNICS |
| Optic Size | S50003C - 5.00 mm,S55003C - 5.50 mm,S6003H2C - 6.00 mm |
| Power/Resolution | All Diopter ranges are available |
| Model Name/Number | S50003C,S55003C,S6003H2C |
| Available Powers | +10.0 D to +35.0 D in 0.5 D steps,-5.0 D to +9.0 D & +31.0 to +35.0 D in 1.0 D Steps |
| Sterilization | Yes |

Features & Key Advantages:

Larger optic diameter.

Step-vaulted haptic design continuous posterior surface contact with the capsular bag to prevent PCO.

Increased stability.

Clear vision.

Optics are YAG Laser Compatible.

Sterilized by Ethylene Oxide.

CATEGORY: OPHTHALMIC KNIVES

Online source: https://www.globalophthalmic.com/ophthalmic-knives.html

32. Ophthalmic Surgical Knife

Indicative Price: Approx. Rs 45 / Piece   | High-Res Image: https://5.imimg.com/data5/SELLER/Default/2025/2/488952392/TR/AW/QK/4238712/ophthalmic-surgical-knife-500x500.jpg

Description: An ophthalmic surgical knife is a precision instrument designed for delicate eye surgeries, such as cataract removal, corneal transplants, and refractive procedures. Typically made from high-quality stainless steel or other durable materials

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Angle of Blade | 0 Degree |
| Material | Stainless Steel |
| Tip Shape | Straight |
| Size | 0 to 1.5mm |
| Usage/Application | Hospital |

CATEGORY: IOL DELIVERY SYSTEM

Online source: https://www.globalophthalmic.com/iol-delivery-system.html

33. IOL Delivery System

Indicative Price: Approx. Rs 150 / Piece   | High-Res Image: https://4.imimg.com/data4/TR/ST/MY-4238712/iol-delivery-systems-500x500.jpg

Description: GLOJECT
single use injector system is specially designed for smoother and safer delivery of acrylic foldable IOL.
Gloject injector system delivers the IOL from 1.8 mm to 2.8 mm incision cartridges.Click-lock mechanism thereby securing the lens in the exact position for a perfect lens delivery into the eye.
Lens insertion is gentle and without the lens wedging or twisting ensures a controlled release, and consequently a precise positioning in the eye.
Gloject injector and cartridge sterilized by Ethylene Oxide.

Specifications Detail Table:

| Specification Parameter | Value / Details |
| --- | --- |
| Usage/Application | Laboratory |
| Brand | Global Ophthalmic |
| Country of Origin | Made in India |

Features & Key Advantages:

Sterilized by ethylene oxide.

Precise positionining in the eye.

Securing the lens in the exact position.